Table of Contents

# Product Requirements Document — LCA Carbon Calculator (EN 15978)

| Product | LCA Carbon Calculator — early-stage embodied & operational carbon assessment |
| --- | --- |
| Standard alignment | EN 15978 (building-level assessment) · EN 15804 (product EPDs) · ISO 14040 / 14044 (LCA framework) |
| Platform | Python · Streamlit web app |
| Document type | Technical build specification (PRD) |
| Version | 0.1 (draft) |
| Date | 23 June 2026 |
| Author | Upasana Sen |
| Status | Draft for review |

Scope note on standards: EN 15978 and EN 15804 are copyrighted standards published by CEN; their full normative text is paywalled. This PRD describes the widely-documented lifecycle-module framework defined by those standards, not verbatim normative clauses. Before publishing carbon results externally, the methodology should be checked against a licensed copy of EN 15978:2011 and the relevant national annex.

## 1. TL;DR

The LCA Carbon Calculator is a Streamlit web application that lets architects, engineers, and sustainability consultants estimate the whole-life carbon of a building at the early design stage, when a model is still a rough bill of quantities rather than a detailed BIM file. The user enters building elements and material quantities, the app pulls third-party-verified emission factors from public Environmental Product Declaration (EPD) databases, and it returns global warming potential (GWP, in kg CO₂e) broken down by EN 15978 lifecycle module (A1–A5, B6, C1–C4) and by building element.

The differentiator is transparency and standard-alignment: every number is traceable to a named data source and a named lifecycle module, and the methodology follows the ISO 14040/44 LCA framework. The target outcome is to let a design team compare two structural or material options on carbon in minutes, not weeks.

## 2. Problem statement

Embodied carbon — the emissions locked into materials and construction before a building is even occupied — is decided early in design but measured late, if at all. Whole-building LCA today is typically performed by specialist consultants near the end of design development, using desktop tools that require a near-complete BIM model. By then the high-leverage decisions (structural system, primary materials, massing) are frozen, so the assessment documents the carbon rather than reducing it.

The people making those early decisions — architects and engineers in concept and schematic design — rarely have a fast, credible way to test "what if we used X instead of Y?" against a recognised standard. The cost of not solving this is locked-in embodied carbon that cannot be undone, weaker competitive positioning as procurement increasingly asks for carbon data, and reliance on expensive late-stage consulting.

## 3. Background: the standards (primer)

This section is build context, not requirements. It defines the vocabulary the rest of the spec uses.

ISO 14040 / ISO 14044 define the general framework for Life Cycle Assessment: goal & scope definition, inventory analysis, impact assessment, and interpretation. They are method-agnostic and apply to any product. Our calculator implements a cradle-to-grave (with optional cradle-to-gate) assessment consistent with this framework.

EN 15804 is the European standard governing product-level EPDs for construction products. EPDs declare environmental impacts (including GWP) per a defined functional/declared unit (e.g. per kg, per m³, per m²). EPDs are the emission-factor data source for this tool.

EN 15978 is the European standard for assessing the environmental performance of a whole building. It organises a building's life cycle into information modules:

| Module | Stage | Description |
| --- | --- | --- |
| A1 | Product | Raw material supply |
| A2 | Product | Transport (to manufacturer) |
| A3 | Product | Manufacturing |
| A4 | Construction | Transport to site |
| A5 | Construction | Construction / installation (incl. material wastage) |
| B1 | Use | Material use / emissions in use |
| B2 | Use | Maintenance |
| B3 | Use | Repair |
| B4 | Use | Replacement |
| B5 | Use | Refurbishment |
| B6 | Use | Operational energy use |
| B7 | Use | Operational water use |
| C1 | End of life | Deconstruction / demolition |
| C2 | End of life | Transport (to waste processing) |
| C3 | End of life | Waste processing |
| C4 | End of life | Disposal |
| D | Beyond boundary | Reuse / recovery / recycling benefits & loads |

A1–A3 are almost always reported together (the "cradle-to-gate" product stage) because EPDs declare them as a bundle.

Modules in scope for this product (bolded above): A1–A3, A4, A5, B6, C1–C4. Modules B1–B5, B7, and D are out of MVP scope (see Non-Goals).

## 4. Goals

- Enable a credible early-stage carbon comparison. A user can model two design options and see the GWP delta by module and element in a single session (target: full comparison in < 15 minutes for a 20-line bill of quantities).

- Make every result traceable. 100% of reported GWP values link back to a named data source (EPD or database record) and a named EN 15978 module — no "black box" totals.

- Cover whole-life carbon, not just embodied. Report embodied (A, C) and operational energy (B6) so totals reflect the dominant lifecycle drivers for the chosen study period.

- Reduce dependence on specialist tooling for screening. Give non-specialists a defensible first-pass number without a full BIM model or a consultant.

- Produce a shareable, standard-aligned output. Export a results summary (CSV + PDF/HTML) structured by EN 15978 module suitable for a design review.

## 5. Non-goals

- Not a certified/compliance LCA tool (v1). Output is a screening estimate for decision support, not a verified EN 15978 assessment for green-building certification (e.g. LEED, BREEAM) or regulatory filing. Why: verification and audited datasets are a large separate effort; early-design screening delivers most of the value first.

- No modules B1–B5, B7, or D in v1. Maintenance, replacement, refurbishment, operational water, and end-of-life recovery credits are excluded. Why: they require service-life and scenario data rarely available at concept stage; A/B6/C capture the largest share of whole-life carbon for most buildings.

- No BIM/IFC import in v1. Input is manual or CSV upload, not a parsed Revit/IFC model. Why: robust BIM parsing is its own engineering track; the early-design user often has no model yet.

- Not a cost estimator. Carbon only, no $ / embodied-cost coupling. Why: keeps the data model and scope focused.

- No multi-user accounts / collaboration in v1. Single-session, single-user. Why: avoids backend/auth complexity until the calculation core is proven.

## 6. Target users & personas

| Persona | Role | Need | Sophistication |
| --- | --- | --- | --- |
| Asha — Design architect | Concept/schematic architect | Quick "which option is greener" answers during design | Low LCA knowledge; high design context |
| Ravi — Structural / sustainability engineer | Engineer or in-house sustainability lead | Defensible early numbers, module-level breakdown, sensitivity to material choice | Medium-high; understands EPDs and modules |
| Maya — Sustainability consultant | External consultant doing rapid screening before a full LCA | Fast triage, traceable sources, exportable summary | High; will scrutinise data provenance |

Primary persona for v1: Ravi (enough domain literacy to value module-level transparency, enough volume of decisions to need speed). Asha drives the UX simplicity bar; Maya drives the traceability bar.

## 7. User stories

Grouped by persona, ordered by priority within group.

Project setup

- As an engineer, I want to create a project with a name, gross floor area, study period (years), and location so that operational and transport calculations have the right basis.

- As an architect, I want to start from a template (e.g. "concrete frame office") so that I am not facing a blank sheet.

Inventory entry

- As an engineer, I want to add building elements (e.g. substructure, frame, façade) and assign materials with quantities so that the model mirrors how I think about the building.

- As a user, I want to upload a bill of quantities as CSV so that I do not retype an existing take-off.

- As a user, I want to enter a quantity in a unit I have (kg, m³, m²) and have the app reconcile it with the EPD's declared unit so that I am not doing manual unit math.

Emission factors

- As a consultant, I want to search a connected EPD database for a material and pick a specific EPD so that my factor is traceable to a real declaration.

- As an engineer, I want a sensible generic/default factor when no specific EPD is selected so that I can still get a first number, clearly flagged as generic.

- As a user, I want to override any factor manually and record a note/source so that I can use a value from a supplier datasheet.

Calculation & results

- As an engineer, I want GWP broken down by EN 15978 module and by element so that I can see where the carbon is.

- As an architect, I want to duplicate a project as "Option B", change a few materials, and see the delta so that I can compare designs.

- As a user, I want to see which inputs are generic vs. EPD-specific so that I know how much to trust the total.

Export

- As a consultant, I want to export results as CSV and a formatted report so that I can drop them into a design review.

Edge / error cases

- As a user, when an EPD database is unreachable, I want the app to fall back to cached/generic factors and tell me clearly so that I am not blocked.

- As a user, when a quantity or unit is missing/invalid, I want an inline validation error so that totals are never silently wrong.

## 8. Functional requirements

Prioritised P0 (must-have / MVP), P1 (fast follow), P2 (future, design for it).

### 8.1 Project & inventory management

P0

- Create / edit / delete a project with: name, building type, gross floor area (m²), study period (years, default 60), location (country/region for energy grid + transport defaults).

- Add elements grouped by a standard element taxonomy (substructure, superstructure/frame, envelope, internal finishes, services — a simplified Uniclass/RICS-style grouping).

- Add line items: material, quantity, unit (kg / m³ / m² / each), optional density (to convert volume↔mass), optional transport distance (A4), optional wastage factor (A5).

- Duplicate a project to create design options for comparison.

Acceptance criteria

- ☐ Given a project with ≥1 valid line item, when the user requests results, then a total GWP and per-module breakdown are produced.

- ☐ Given a line item in m³ and an EPD declared per kg, when a density is provided, then the app converts correctly and shows the converted mass.

- ☐ Given a missing required field (quantity or unit), then the line item is flagged and excluded from totals with a visible warning.

P1

- CSV bill-of-quantities upload with column mapping.

- Project templates (pre-filled archetypes).

P2

- IFC/BIM quantity import.

- Element-level service life for B4 replacement modelling.

### 8.2 Emission-factor data (EPD integration)

P0

- Integrate at least one open EPD data source (see §11). Search by material name; display candidate EPDs with: declared unit, GWP A1–A3, geography, validity date, source.

- Allow selection of a specific EPD record; persist the chosen record's identifier with the line item.

- Provide a bundled set of generic/default factors for common materials so the app works offline / before a database pick, clearly labelled "generic".

- Manual factor override with a free-text source note.

Acceptance criteria

- ☐ Given a material search, when the database returns results, then each result shows GWP value, unit, and a source attribution.

- ☐ Given an EPD is selected, when results are computed, then the line item's factor, unit, and source match the selected EPD.

- ☐ Given the database is unavailable, then the app uses cached/generic factors and shows a non-blocking "using generic data" notice.

- ☐ Given a manual override, then the override value and note are used and visibly marked as user-supplied.

P1

- Cache EPD lookups locally to reduce API calls and enable offline reuse.

- Multiple data sources with source preference order and de-duplication.

P2

- Background refresh of cached EPDs on validity expiry; alert on expired EPDs.

### 8.3 Calculation engine

P0 — Compute GWP per module per line item, aggregate to element and project level. Modules in scope: A1–A3, A4, A5, B6, C1–C4. See §10 for the methodology and formulas.

Acceptance criteria

- ☐ Module math matches the formulas in §10 for a documented worked example (unit-tested).

- ☐ Changing a quantity, factor, distance, or wastage value updates totals deterministically.

- ☐ Totals are reproducible: identical inputs always yield identical outputs (no hidden randomness; pinned factor data).

P1 — Sensitivity view: show contribution % by element/material; highlight top carbon drivers.

P2 — Uncertainty ranges (e.g. low/expected/high factor bands); Monte Carlo on factor uncertainty.

### 8.4 Results, comparison & export

P0

- Results dashboard: total GWP (kg CO₂e and kg CO₂e/m²), breakdown by module (stacked bar), breakdown by element (bar/table), embodied vs operational split.

- Side-by-side comparison of two projects/options with absolute and % delta.

- Export results to CSV; export a formatted summary (HTML or PDF) structured by module.

Acceptance criteria

- ☐ Given results exist, then the dashboard shows module and element breakdowns whose parts sum to the displayed total (within rounding).

- ☐ Given two options, then comparison shows per-module deltas and an overall delta.

- ☐ Given an export request, then the CSV contains every line item with material, quantity, unit, factor, source, module GWPs, and totals.

P1 — Save/load projects to disk (JSON); shareable export bundle.

P2 — Benchmark against published GWP/m² ranges by building type (requires a vetted benchmark dataset — flagged as needing a real source).

## 9. Non-functional requirements

- Transparency: every displayed figure must be drillable to its inputs (quantity × factor, with module and source). This is a product principle, not a nice-to-have.

- Reproducibility: factor datasets are versioned/pinned; a result records which dataset version produced it.

- Performance: recalculation of a ≤200-line project in < 1 s after an input change; EPD search response surfaced in < 3 s (network-dependent; show a spinner and allow generic fallback).

- Resilience: no external dependency may be a single point of failure for producing a number — generic factors always allow a fallback estimate.

- Accuracy honesty: the UI must distinguish EPD-specific vs generic vs user-override factors, and must never present a generic-data estimate as if it were EPD-verified.

- Portability: runs locally with streamlit run and deploys to Streamlit Community Cloud with no paid infra.

## 10. Calculation methodology

All impacts are Global Warming Potential (GWP100) in kg CO₂e. The general LCA relationship per line item is:

module_GWP = quantity_in_declared_unit × emission_factor_for_module

Quantities are normalised to the EPD's declared unit before multiplication (mass↔volume via density; area via thickness/areal density where needed).

### A1–A3 — Product stage (cradle-to-gate)

GWP_A1A3 = mass_or_declared_qty × factor_A1A3

factor_A1A3 comes directly from the selected EPD (or generic default). This is the core embodied figure.

### A4 — Transport to site

GWP_A4 = mass_tonnes × distance_km × transport_emission_factor (kgCO2e per tonne-km)

Transport factor depends on mode (road/rail/sea). Provide editable defaults by mode; default distance configurable per project.

### A5 — Construction / installation (incl. wastage)

Two components:

GWP_A5_waste = GWP_A1A3 × wastage_rate            # embodied carbon of wasted material
GWP_A5_site  = (site energy / process factor)      # optional, often small at early stage

MVP models wastage (material-specific default rate, editable). Site-process energy is optional/editable, default 0 with a note.

### B6 — Operational energy use

GWP_B6 = annual_energy_use (kWh/yr) × grid_emission_factor (kgCO2e/kWh) × study_period_years

annual_energy_use is entered directly or estimated from floor area × an editable energy-intensity (kWh/m²/yr). grid_emission_factor defaults by location and is editable. Grid factors fall over time and vary by country — these defaults must be sourced from an official dataset and dated (flagged; do not hard-code unsourced values).

### C1–C4 — End of life

GWP_C = mass_or_declared_qty × factor_C(module)

End-of-life factors (per module C1–C4) come from the EPD where declared; otherwise generic scenario factors by material/disposal route. Where an EPD reports C as a combined value, store it against C and note the aggregation.

### Aggregation

embodied_GWP    = Σ (A1A3 + A4 + A5 + C1..C4) over all line items
operational_GWP = Σ B6
total_GWP       = embodied_GWP + operational_GWP
intensity       = total_GWP / gross_floor_area      # kgCO2e/m²

### Worked example (illustrative factors — NOT real data)

The numbers below are placeholders to validate the math, not sourced emission factors. Replace with EPD/database values before any real use.

- 100,000 kg of a material; factor_A1A3 = 0.12 kgCO2e/kg (illustrative) → A1–A3 = 12,000 kg CO₂e

- Transport 50 km at 0.0001 kgCO2e/(kg·km) (illustrative) → A4 = 100,000 × 50 × 0.0001 = 500 kg CO₂e

- Wastage 5% → A5 = 12,000 × 0.05 = 600 kg CO₂e

- End of life factor_C = 0.02 kgCO2e/kg (illustrative) → C = 2,000 kg CO₂e

- Embodied for this item = 12,000 + 500 + 600 + 2,000 = 15,100 kg CO₂e

Each step must be covered by a unit test using these documented inputs.

## 11. EPD data source integration

The "comprehensive" data strategy is to read emission factors from external, third-party-verified EPD databases via API, with a bundled generic dataset as a guaranteed fallback. Two complementary public sources are recommended for v1.

Verify before building: API endpoints, authentication, and access terms below were confirmed from public documentation as of mid-2025 and are changing (notably EC3's "2.0" modernization of its API program). Re-check the live developer docs and terms of use before integrating, and confirm attribution requirements.

### 11.1 EC3 / openEPD (Building Transparency)

- What: the largest open-access database of digital, third-party-verified EPDs; GWP reported in kg CO₂e.

- Access model: requires an EC3 account and an API key (created in EC3 Key Management). Free API access for individuals affiliated with a verified organization/institution; paid tier required for production use. Building Transparency asks to be notified before integrating EC3 data, and attribution to Building Transparency/EC3 is required.

- Endpoints (verify): openEPD API base https://openepd.buildingtransparency.org/api; an EC3 materials endpoint historically at https://etl-api.cqd.io/api/materials. Querying uses an "Open Material Filter" (OMF) query language; responses can be filtered to return fields such as gwp.

- Why for us: breadth, global coverage, active ecosystem (Autodesk, cove.tool, etc. integrate it), strong provenance — satisfies Maya's traceability bar.

### 11.2 ÖKOBAUDAT (German Federal Ministry, via soda4LCA)

- What: government-maintained LCA/EPD database; ~1,400+ datasets in German and English; GWP in kg CO₂e with explicit reference units (kg, m³, pcs, MJ). Data is publicly available free of charge.

- Access model: RESTful soda4LCA service interface. Data is in the EC-derived ILCD format (extended for EN 15804). Return format defaults to XML; pass format=JSON for JSON. An "extended view" returns all referenced secondary data inline in one request.

- Why for us: fully open, no auth gate, stable government stewardship — ideal default source and a clean fallback when EC3 access is constrained.

### 11.3 Integration design

- A data-source adapter layer with a common internal interface: search(material_query) -> [FactorCandidate] and get(record_id) -> EmissionFactorSet. Each source (EC3, Ökobaudat, bundled-generic) implements the adapter, normalising to our internal EmissionFactorSet (per-module GWP + declared unit + geography + validity + source citation).

- Normalisation: map each source's fields to our module model; record declared unit and convert at calculation time, never at ingest.

- Caching: persist fetched records locally (keyed by source + record id + dataset version) to cut API calls and enable offline reuse; respect EPD validity dates.

- Fallback chain: selected EPD → cached EPD → generic default, with the UI always showing which tier was used.

- Attribution & licensing: store and display source attribution per the source's terms (required for EC3). Document data licences in the repo.

## 12. Data model

Core entities (suitable for in-memory + JSON persistence in v1; portable to a relational store later):

- Project — id, name, building_type, gross_floor_area_m2, study_period_years, location, dataset_version, created_at, options[]

- Element — id, project_id, category (substructure/frame/envelope/finishes/services), name

- LineItem — id, element_id, material_name, quantity, unit, density?, transport_distance_km?, transport_mode?, wastage_rate?, factor_source_type (epd|generic|override), factor_record_id?, override_factors?, notes

- EmissionFactorSet — record_id, source (ec3|okobaudat|generic), declared_unit, gwp_by_module {A1A3, A4?, A5?, C1, C2, C3, C4}, geography, valid_until, citation

- ResultRow — derived: line_item_id, module_gwps{}, item_total

- ProjectResult — derived: embodied_gwp, operational_gwp, total_gwp, intensity_per_m2, breakdown_by_module, breakdown_by_element, data_quality_summary (epd vs generic counts)

## 13. System architecture

Single Streamlit application, layered so the calculation core is independent of the UI and the data sources.

┌────────────────────────────────────────────┐
│ Streamlit UI (app.py + pages/)              │  project setup, inventory, results, compare
├────────────────────────────────────────────┤
│ Service layer                               │  orchestrates: build inventory → fetch factors → calculate
├───────────────┬───────────────┬─────────────┤
│ Calc engine   │ Data adapters │ Persistence  │
│ (pure Python, │ EC3 │ Öko │   │ JSON load/   │
│  unit-tested) │ Generic       │ save, cache  │
└───────────────┴───────────────┴─────────────┘
        │               │
   no I/O, pure     external EPD APIs (HTTP)
   functions

- Calc engine is pure functions (no network, no Streamlit) → fully unit-testable and reproducible. This is the heart of the product and the part that must be correct.

- Data adapters isolate each EPD source behind the common interface; adding a source = adding an adapter.

- Service layer wires inventory + factors + engine and applies the fallback chain.

- Persistence is JSON files locally + an on-disk factor cache; no database required for v1.

### Suggested repository layout

lca-carbon-calculator/
├── app.py                  # Streamlit entry
├── pages/                  # multipage UI (setup, inventory, results, compare)
├── lca/
│   ├── engine.py           # pure calculation functions (A,B6,C, aggregation)
│   ├── models.py           # dataclasses for entities above
│   ├── factors/
│   │   ├── base.py         # adapter interface + FactorCandidate/EmissionFactorSet
│   │   ├── ec3.py          # EC3/openEPD adapter
│   │   ├── okobaudat.py    # soda4LCA adapter
│   │   └── generic.py      # bundled default factors (CSV/JSON)
│   ├── cache.py            # local EPD cache
│   └── persistence.py      # project save/load (JSON)
├── data/
│   └── generic_factors.csv # bundled fallback factors (dated + sourced)
├── tests/
│   └── test_engine.py      # worked-example + module unit tests
├── requirements.txt
└── README.md

## 14. Tech stack

| Concern | Choice | Notes |
| --- | --- | --- |
| Language | Python 3.11+ |  |
| UI | Streamlit | multipage app; st.session_state for in-session model |
| Data handling | pandas | inventory tables, CSV import/export |
| Charts | Streamlit native / Plotly | stacked module bars, element breakdown |
| HTTP | requests (or httpx) | EPD API calls |
| Models | dataclasses / pydantic | typed entities, validation |
| Testing | pytest | engine unit tests are mandatory |
| Persistence | JSON files + on-disk cache | no DB in v1 |
| Deploy | Streamlit Community Cloud | free tier; secrets for API keys via st.secrets |

API keys (EC3) live in st.secrets / environment variables, never in the repo.

## 15. UX / primary flow

Multipage Streamlit app:

- Project setup — name, building type, floor area, study period, location. Optional: start from template.

- Inventory — add elements and line items in an editable table (st.data_editor); per-row: material search → pick EPD (or generic/override), quantity + unit, optional density/distance/wastage. Live data-quality indicator (how many rows are EPD-backed vs generic).

- Results — headline total + intensity (kg CO₂e/m²); stacked bar by module; bar/table by element; embodied vs operational split; data-quality summary.

- Compare — pick two options; side-by-side totals and per-module deltas.

- Export — CSV (full line-item detail) + formatted report (HTML/PDF) organised by module.

UX principles: never hide provenance; generic/estimated values visually distinct from EPD-verified; every total drillable to inputs.

## 16. Success metrics

Leading (days–weeks)

- Time to first result for a new user with a 20-line BoQ — target < 15 min.

- % of line items backed by a specific EPD (vs generic) per completed project — target median ≥ 50% as a quality signal.

- Recalculation latency after an input change — target < 1 s for ≤200 lines.

- Task completion: % of started projects that reach a results view.

Lagging (weeks–months)

- Repeat use: projects created per returning user.

- Option-comparison usage: % of projects that create ≥2 options (indicates it's used for decisions, the core value).

- Qualitative: design teams reporting a material/structural change influenced by the tool.

Targets above are hypotheses for a portfolio/MVP context, not benchmarked figures — validate with real usage before treating them as KPIs.

## 17. Risks & mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| EC3 API access terms / endpoints change (2.0 transition) | Integration breaks | Adapter isolation; Ökobaudat + generic fallback; pin to documented API version; re-verify before build |
| Users trust generic estimates as if EPD-verified | Misleading results | Hard UI distinction; data-quality summary; disclaimer on exports |
| Unit mismatches (declared unit vs entered unit) | Silent calculation errors | Explicit unit reconciliation + density; unit tests; validation that blocks ambiguous rows |
| Operational (B6) dominates and swamps embodied signal | Misreads design levers | Always show embodied vs operational split separately; allow B6 = 0 toggle for embodied-only screening |
| Out-of-date grid / transport factors | Inaccurate A4/B6 | Source defaults from official datasets, store the date, make them editable, show the source |
| Scope creep into a full certified LCA | Never ships | Explicit non-goals; phase gates; keep engine modular |

## 18. Open questions

- [Data/Legal] What are EC3's current production API terms and rate limits under the 2.0 program, and do attribution/notification requirements allow a public portfolio deployment? (blocking for EC3 integration; non-blocking overall — Ökobaudat can ship first)

- [Data] Which official source and vintage should supply default grid-electricity emission factors per region? (blocking for B6 defaults)

- [Methodology] For materials whose EPD reports end-of-life as a single combined C value, how should it be displayed — combined C, or apportioned to C1–C4? (non-blocking; default to combined + note)

- [Methodology] Default study period — 60 years assumed; confirm against the convention the target users expect. (non-blocking)

- [Product] Which element taxonomy (RICS / Uniclass / simplified custom) best fits the early-design user without overwhelming them? (non-blocking; start simplified)

- [Data] Source for any GWP/m² benchmark ranges (P2 feature) — needs a vetted, citable dataset before building. (deferred)

## 19. Roadmap / phasing

Phase 1 — Calculation core (P0 engine + models) Pure engine for A1–A3, A4, A5, B6, C1–C4 + aggregation; dataclasses; bundled generic factors; full unit tests on the worked example. Ships value even with manual factors.

Phase 2 — Streamlit MVP (P0 UI + one data source) Project setup, inventory editor, results dashboard, Ökobaudat (open, no-auth) integration, generic fallback, CSV export.

Phase 3 — Comparison + EC3 + caching (P0/P1) Option duplication & side-by-side compare; EC3/openEPD adapter; local EPD cache; data-quality summary; formatted report export.

Phase 4 — Polish & fast-follows (P1) CSV BoQ upload, templates, sensitivity view, project save/load.

Future (P2) — BIM/IFC import, B1–B5 service-life modelling, uncertainty ranges, benchmarking.

## 20. Appendix

### A. Glossary

- GWP / kg CO₂e — Global Warming Potential, the carbon-equivalent impact metric.

- EPD — Environmental Product Declaration; a verified document of a product's environmental impacts (EN 15804).

- Embodied carbon — emissions from materials & construction (modules A, C, and B1–B5); excludes operational energy.

- Operational carbon — emissions from running the building (B6 energy, B7 water).

- Declared/functional unit — the unit an EPD's impacts are stated per (kg, m³, m²…).

- Cradle-to-gate — A1–A3 only. Cradle-to-grave — A–C (and optionally D).

### B. Module coverage summary

In scope (v1): A1–A3, A4, A5, B6, C1–C4. Out of scope (v1): B1–B5, B7, D.

### C. References (verify against live/licensed sources)

- EN 15978:2011 — Sustainability of construction works — Assessment of environmental performance of buildings — Calculation method (CEN; paywalled).

- EN 15804 — Sustainability of construction works — EPDs — Core rules for the product category of construction products (CEN; paywalled).

- ISO 14040:2006 / ISO 14044:2006 — Environmental management — Life cycle assessment (ISO; paywalled).

- EC3 / Building Transparency — tool, API access & pricing, and openEPD docs: buildingtransparency.org, docs.buildingtransparency.org, openepd.buildingtransparency.org.

- ÖKOBAUDAT — oekobaudat.de (data + software-developer guidance); soda4LCA service (soda4lca.io).

All standard clause details and emission-factor values must be confirmed against licensed standards and official EPD/database records before any non-screening use.